SUBJECT:			Derating Curve for 34044-1

author:			larry lim

TECHNICIAN:		larry lim

REPORT NO:		ENG-14-SEPT-2020-1

date:			09/14/2020

- Part Description:

34044-1 Rev C PCB Assembly

SW: 010E

- Equipment List:

High current test fixture. 80TK Thermocouple Module, Multimeter, Fluke 61 IR Thermometer, Espec Test Chamber

- Test Setup:

|  | Unit is first run at ambient. Total temperature increased is subtracted from 80°C to be used as a starting point for derating curve. Next point on curve would be to start with a minimum load and slowly increase the load until temperature reaches 80°C All PWM outputs are loaded with 3A. |
| --- | --- |

- Results:

| Ambient: 23.6°C Load Current: 33A Highest Temperature Recorded: 44 °C Total Temperature Increase: 20.4°C 59.6°C starting point for derating curve. |  |
| --- | --- |
| Set temperature: 73°C Load Current: 26A Highest Temperature Recorded: 79.2°C Total Temperature Increase: 6.2°C |  |

- Reference Data: